# ArduPilot Coding Research

Pi 5 test application for vertical takeoff and landing. Pixhawk runs ArduCopter control. Custom chassis, 100 g payload. Bullets include proposed implementation tasks.

# Environment and connections

Pymavlink Python guide

Use 64-bit Raspberry Pi OS and a Python virtual environment.

Install pymavlink; pin the tested library, Python, and ArduCopter versions.

Import mavutil; create a mavlink_connection() for UDP or serial.

Wait for a heartbeat with a deadline; verify autopilot and system identity.

Use discovered target IDs and ArduPilot dialect definitions.

Proposed stack: direct pymavlink control with Mission Planner monitoring.

ArduPilot SITL

Run SITL on a development computer after installing its build environment.

Launch: sim_vehicle.py -v ArduCopter -f quad --console --map.

Forward MAVProxy output to the Pi IP and a chosen UDP port.

Use separate outputs for the application and ground station.

The stock quad validates sequencing; custom performance needs a matched model.

Raspberry Pi UART configuration

Choose USB serial or a configured 3.3 V UART connection.

Cross TX/RX, share ground, and verify flight-controller connector pinouts.

Pi 5 primary UART defaults to its debug header; select the intended interface.

Disable serial console use; check device permissions and stable device paths.

Store transport, device path, and baud in configuration.

ArduPilot Raspberry Pi connection

Configure the connected port for MAVLink 2; SERIALx_PROTOCOL = 2.

Match SERIALx_BAUD to the Pi; verify the board’s logical port mapping.

Older Pi device-path and power examples require Pi 5-specific checking.

Power the Pi from an appropriate regulator, not the TELEM supply.

Pi sends high-level commands; ArduCopter controls ESC outputs.

# Test sequence and telemetry

Copter Guided commands

States: connect → ready → GUIDED → arm → take off → hover → LAND → disarmed.

Arm with MAV_CMD_COMPONENT_ARM_DISARM; verify the armed heartbeat flag.

Send MAV_CMD_NAV_TAKEOFF with target altitude above home in param7.

Confirm climb from telemetry; command acceptance does not prove motion.

Replace fixed sleeps with measured conditions and deadlines.

Log transitions, command results, altitude, drift, attitude, battery, and mode.

Prearm checks

Keep flight arming checks enabled; resolve reported failures.

Capture STATUSTEXT and verify calibration, battery, and estimator readiness.

Require valid horizontal positioning for the planned Guided test.

Abort before arming if readiness or fresh telemetry is missing.

Copter Land mode

Send MAV_CMD_NAV_LAND or request LAND mode; verify the actual mode.

Check landing-speed parameters and units for the installed firmware.

Watch landed state, vertical motion, and armed status.

Require actual touchdown and disarming before reporting success.

Taut restraints can distort landing detection; allow real gear contact.

Optical flow setup

Indoor option: optical flow plus range sensing and configured EKF sources.

A rangefinder alone cannot establish horizontal position.

Validate EKF origin and local-position quality before Guided flight.

Use consistent coordinates; LOCAL_POSITION_NED uses positive down.

Measure climb relative to start; detect estimator resets or jumps.

# Reliability and verification

Requesting telemetry

Use MAV_CMD_SET_MESSAGE_INTERVAL to request individual message rates.

Request HEARTBEAT, LOCAL_POSITION_NED, ATTITUDE, and BATTERY_STATUS as needed.

Request EXTENDED_SYS_STATE for landed state where supported.

Check actual arrival rates; ground-station stream requests can interfere.

Proposed: one receive loop dispatches messages to telemetry and ACK handlers.

MAVLink command protocol

Match COMMAND_ACK to the outstanding command and sending vehicle.

Handle accepted, rejected, in-progress, and timeout results explicitly.

Verify actual mode and armed state independently of command responses.

Use bounded retries; avoid duplicate takeoff requests after ambiguous results.

Python concurrency and timeouts

Run sequencing, telemetry, and watchdogs concurrently.

Keep blocking pymavlink reads off the asyncio loop; use a worker and queue.

Use monotonic deadlines and per-message freshness checks.

Proposed modules: connection, telemetry, sequence, watchdog, logger.

Cancelling Python does not stop the aircraft; implement tested recovery.

Copter GCS failsafe

Configure FS_GCS_ENABLE, FS_GCS_TIMEOUT, and relevant failsafe options.

Send periodic application heartbeats; verify which sender the firmware monitors.

Confirm a surviving GCS or router cannot mask a dead control application.

Test Pi termination, link loss, and stale data in SITL before hardware.

Choose a fixture-compatible response; keep independent RC takeover available.

ArduPilot flight logs

Save onboard DataFlash logs, MAVLink telemetry, and application CSV or JSON.

Archive parameters, firmware version, code commit, mass, and battery state.

Compare target/measured height, XY drift, attitude, and motor outputs.

Proposed pass: sequence completes within limits, without faults, then disarms.

Advance: SITL → prop-free integration → supervised restrained flight.

Require three consecutive passing runs before expanding test limits.

Guided setpoints

Optional later step: SET_POSITION_TARGET_LOCAL_NED with explicit type masks.

Provide all three axes for each enabled position, velocity, or acceleration group.

Resend velocity/acceleration commands at least once per second.

Check GUID_TIMEOUT; Guided_NoGPS provides attitude control, not XY holding.