MAVSDK Python QuickStart

Use 64-bit Raspberry Pi OS and a Python virtual environment.

Current MAVSDK supports Linux aarch64 wheels, including 64-bit Pi OS.

Install mavsdk; pin the tested package version and Python version.

Current async imports use mavsdk.asyncio and ActionAsync.

Connect, discover the autopilot, then create its action plugin.

MAVSDK 3.x examples use a different API; keep code and docs aligned.

PX4 Gazebo simulation

Run PX4 SITL and Gazebo on a development computer.

Launch: make px4_sitl gz_x500.

Connect the Pi test application over the network; monitor in QGroundControl.

SITL commonly sends SDK traffic to UDP 14540; configure the Pi endpoint.

Stock X500 validates sequencing; model the custom chassis for performance.

Raspberry Pi UART configuration

Choose USB serial or a configured 3.3 V UART connection.

For GPIO UART: cross TX/RX, share ground, and verify connector pinouts.

Pi 5 primary UART defaults to its debug header; select the intended interface.

Disable serial console use; match baud and check device permissions.

Store transport, device path, and baud in configuration.

Pixhawk connection reference

Configure the selected flight-controller port for MAVLink.

Verify heartbeat and vehicle identity before sending commands.

Pi sends high-level commands; Pixhawk drives ESC outputs.

Keep power regulation separate from the data connection.

# Test sequence and telemetry

MAVSDK takeoff and landing example

Legacy API example: use its sequence, or pin its compatible SDK.

States: connect → ready → arm → climb → hover → land → disarmed.

Confirm each transition through fresh telemetry.

Command acceptance does not prove altitude or touchdown.

Replace fixed sleeps with measured conditions and deadlines.

Adapt global-position readiness checks for validated indoor local positioning.

PX4 Takeoff mode

Automatic takeoff requires valid local position.

Configure target altitude and climb speed within fixture clearance.

Review MIS_TAKEOFF_ALT and MPC_TKO_SPEED.

Proposed climb check: height reaches target tolerance before timeout.

Proposed hover check: height, drift, and attitude stay within agreed limits.

PX4 Land mode

Land descends at the location where the mode starts.

Review MPC_LAND_SPEED and COM_DISARM_LAND.

Watch landing state, vertical motion, and armed status.

Require touchdown confirmation and disarming before reporting success.

On timeout, record failure and invoke the tested recovery procedure.

PX4 optical flow integration

Indoor option: downward optical flow plus distance sensing.

A rangefinder alone cannot establish horizontal position.

Check local-position validity before arming and throughout flight.

Use one consistent local frame; NED down is positive.

Compute climb from the starting position; detect estimator resets.

Log height, XY drift, attitude, battery, mode, link, and landing state.

# Reliability and verification

Python asyncio tasks and timeouts

Run telemetry collection, sequencing, and watchdogs concurrently.

Use asyncio.wait_for() for bounded waits; use a monotonic clock.

Timestamp samples; reject stale data even while commands succeed.

Catch command errors; record state and reason before recovery.

Use try/finally for cleanup; cancelling Python does not stop the aircraft.

Proposed modules: connection, telemetry, sequence, watchdog, logger.

PX4 failsafes

Configure PX4 link-loss, position-loss, and battery responses.

A dead Pi cannot send a recovery command; PX4 must handle that case.

Keep RC takeover and emergency motor stop independent of Pi.

Test kill behavior without props; releasing within five seconds can restart motors.

Proposed config: altitude, speed, drift/tilt limits, timeouts, sample-age limits.

PX4 failsafe simulation

Test missing readiness, rejected commands, and unreachable target altitude.

Test telemetry loss, Pi process termination, and position failure in SITL.

Check both application decisions and the actual PX4 failsafe response.

Advance: SITL → prop-free integration → supervised restrained flight.

Keep restraints unloaded in hover; allow actual landing-gear touchdown.

PX4 Flight Review

Save PX4 ULog plus timestamped application CSV or JSON logs.

Record commands, responses, transitions, telemetry, and failure reasons.

Archive PX4 parameters, SDK version, code commit, mass, and battery state.

Compare commanded height with measured height, drift, and motor outputs.

Proposed pass: complete sequence, no faults, touchdown, and disarming.

Require three consecutive passing runs before expanding test limits.

PX4 Offboard mode

Optional later step for explicit vertical setpoints; start with Takeoff/Land.

PX4 requires a >2 Hz proof-of-life stream, established for over one second.

Check COM_OF_LOSS_T and configured offboard-loss action.

Validate setpoint frame, sign, and loss response in SITL first.